Analysis and Experimental Validation of Simultaneous and Distributed Diagnosis of Wire Networks

Mouad Addad, Ali Djebbari, Evan Benoit, and Cynthia M. Furse, Fellow, IEEE

Abstract— Reflectometry-based techniques such as Spread Spectrum and Sequence Time Domain Reflectometry (SS/STDR) have been extensively used for the detection, localization, and characterization of electric faults in wires. However, in branched wire networks, testing using a single sensor suffers from ambiguity where it can be difficult to determine which branch contains the fault. Distributed reflectometry, which uses multiple sensors to test the network from different locations, can resolve this ambiguity. This paper evaluates pseudo-noise (PN) codes for simultaneous distributed testing, Maximum length (ML), Gold and zero correlation zone (ZCZ) codes are compared for a set of up to 16 simultaneous sensors. ML and Gold codes show significant interference between sensors, but the ZCZ codes show near-zero interference over their measurement zone. This lack of interference greatly enhances their use for locating small faults. The results were verified numerically and validated experimentally. Index Terms— spread spectrum time domain reflectometry (SSTDR), spectral time domain reflectometry (STDR), distributed reflectometry, non-destructive evaluation, fault location, wire networks, simultaneous testing, continuous diagnosis.

# Introduction

Wired networks are ubiquitous wherever the transfer of energy and information is needed. They can be found in aircraft, cars, buildings, railways, nuclear power plants, and more [1]. Electrical faults in these systems plague maintainers, driving the need for health monitoring tools able to detect and accurately locate faults in wired networks. Reflectometry is a powerful technique that has been used extensively for the detection, localization, and characterization of electric faults  [3] [4]. Reflectometry consists of injecting a test signal into the wire. A part of its energy is reflected back to the injection point when it crosses impedance discontinuities within the wire. The test device then extracts information about the location and the nature of these discontinuities from the reflected signal. If the topology of the network is branched, testing using a single device suffers from ambiguity.  If a reflectometry sensor is testing a wire network with two equal-length arms, one of which has a fault, it is impossible to determine which of the arms is faulty from this single sensor.  [5]. If an open circuit and a short circuit, which have equal and opposite reflections appear at the same distance from the sensor, their reflections cancel each other out and cannot be located from a single sensor. Furthermore, because the test device impedance is not matched to the characteristic impedance of the wire, a part of the energy of the injected test signal is immediately reflected to the reflectometer, giving rise to a phenomenon called a “blind zone” in which faults close to the injection point cannot detected. Another challenge for branched networks is that each branch reflects some of the transmitted thus reducing the reflection that returns to the tester and limiting the number of branches a single tester can see through before the signal is below its detection threshold. Attenuation on long wire lengths exacerbates this problem, To solve these problems, distributed reflectometry uses multiple reflectometry-based sensors placed at various ends of the wire network [6]. Each sensor performs its diagnosis, providing a different perspective of the network topology. The aggregation of information from all sensors allows the removal of fault location ambiguities, blind zones, and allows testing of longer and more complex networks. This can be accomplished in two ways. At first glance, activating each sensor one at a time may seem like a good way to prevent interference between them. However, this means that only a portion of the system is being tested at any given time, and intermittent faults will be missed during this time. For continual testing and monitoring, for which S/SSTDR is known, each sensor should test continually and simultaneously. Therefore, specific signal processing is needed to reduce the interfering signals coming from the other sensors [6].

In this paper, an improved method for the simultaneous diagnosis of wire networks based on distributed reflectometry is presented. The main contribution of this work is the complete cancelation of interference between sensors without the recourse to signal processing algorithms. The results obtained were verified numerically and validated experimentally.

This paper is organized as follows. Section II describes the analytical model of the distributed diagnosis system. Section III discusses the selection of suitable signals that provide interference-free measurements. Numerical analysis is carried out in Section IV to verify the analytical results. Experimental validation is conducted in Section V. Concluding remarks are given in Section VI.

# System Model

A block diagram of a Sequence Time Domain Reflectometry (STDR) system for testing a wire network from multiple locations (distributed sensing) (need to show ith sensors in the figure, and instead of Seq. Gen, maybe put STDR. Do we want to include SSTDR also? ) is shown in Fig. 1.

Seq. Gen.

Cable

Correlator

Wire

Network

Fig. 1.  Block diagram of an S/SSTDR system. Copy wire network from fig 3)

The STDR test signal consists of a square wave modulated by a sequence as in

| sit=n=-∞∞si,n pt-nTc | (1) |
| --- | --- |

where sequence si,n∈-1,1 has a chip width, Tc, and the rectangular pulse waveform pt is defined as

| pt=10 | 0≤t≤Tc | (2) |
| --- | --- | --- |
|  | otherwise |  |

Note that sit is a periodic signal with period T=NTc , where N is the sequence length, so that sit=sit+T. After transmission onto the wire network, the received signal at the ith sensor is

| rt= | aosit+papsit-τp |  |
| --- | --- | --- |
|  | +k=1k≠iKbkskt-τk+dt+nt | (3) |

where ao is the amplitude of the reflection at the testing point due to the impedance mismatch between the measure device and the wire, ap and τp are, respectively, the attenuation coefficients and delays of the test signal sit coming back from multiple paths in the wire network, dt is the  data signal on the network, bk and τk are, respectively, the attenuation coefficients and delays of a different test signal, skt, coming from the kth sensor, and nt is the noise signal.

The reflected signal carries information about the status of the wire network, and many algorithms are available to extract the network topology, detect and locate faults, etc. [ref]. Interference caused by the signals from all sensors transmitting in parallel, data signals, and noise can perturb the received signals. Since the data signal amplitude is usually much greater than that of the test signal, it can be filtered out from the received signal [7]. The effects of noise can be reduced through averaging and other de-noising techniques [3]. In this paper, we will focus on reducing interference from multiple sensors transmitting and receiving simultaneously.

For S/SSTDR reflectometry, the received signal is cross correlated with a reference signal, typically the incident signal sent into the wire network. Since our signal is periodic, the periodic cross correlation function is used:

| Rrsi'τ=1T0Tsi't rt+τ dt | (4) |
| --- | --- |

where si't is the reference signal. The delay τ is proportional to the distance d over which the signal is delayed as

| 2 d=v τ | (5) |
| --- | --- |

where v is the velocity of propagation (VOP) on the wire.  By substituting (3) in (4), we obtain the correlation of the received signal

| Rrsi'τ= | aoRsisi'τ+pap Rsisi'τ-τp |  |
| --- | --- | --- |
|  | +k=1k≠iKbkRsksi'τ-τk | (6) |

Eq. (6) shows that the correlator’s output depends on the correlation properties of sequences. To further clarify this point, let’s rewrite the first term of (6) as follows

| Rsisi'τ=m=0N-1n=0N-1si,msi,n1T0Tpt-mTcpt+τ-nTc dt |  |  |
| --- | --- | --- |
|  |  | (7) |

The integral in (7) is nonzero only when pt-mTc and pt+τ-nTc overlap. The delay τ=lTc+τϕ, where 0≤τϕ<Tc, is any delay, not necessarily an integer number of chips. Using this substitution, the pulses overlap only for n=m+l and n=m+l+1, so that (7) becomes

| Rsisi'τ= | 1-τϕTcEsisi'l+τϕTcEsisi'l+1 | (8) |
| --- | --- | --- |

The discrete periodic cross-correlation function (PCCF) of two sequences sk and si' is defined as

| Esksi'l=1Nn=0N-1sk,nsi,n+l' | (9) |
| --- | --- |

where the subscript n+l≥N is computed modulo N. When k=i, the PCCF becomes the periodic autocorrelation function (PACF). Similarly, it can also be shown that the autocorrelation of one of the reflections in (6) with a delay p from the wire network is

| Rsisi'τ-τp=1-τϕTcEsisi'l-lp+τϕTcEsisi'l-lp+1 |  |  |
| --- | --- | --- |
|  |  | (10) |

and a similar cross correlation between reference and an interference signal in (6) is

| Rsksi'τ-τk=1-τϕTcEsksi'l-lk+τϕTcEsksi'l-lk+1 |  |  |
| --- | --- | --- |
|  |  | (11) |

Note that in the special case where reference and reflection signals are shifted by an integral number of chips, τ=lTc, (6) becomes

| Rrsi'τ= | aoEsisi'l+pap Esisi'l-lp |  |
| --- | --- | --- |
|  | +k=1k≠iKbkEsksi'l-lk | (12) |

A similar expression was obtained in [8] where Rrsi'τ is dependent on the aperiodic correlation properties. In contrast, (12) depends on the periodic correlation properties. Also, similar results can be obtained for spread spectrum time domain reflectometry (SSTDR), which uses a sine wave (or other function) modulated by the STDR signal. The performance of the diagnostic system is improved if sequences with low cross correlation and high autocorrelation are used.

# Sequences Selection for the Proposed System

According to (12), the nonzero side-lobes of the PACF cause interference of the test signal with itself (self-interference) and the nonzero PCCF causes interference between test signals generated by other sensors in the network (mutual interference). Therefore, the optimal sequence set should have impulse-like PACF (zero side-lobes) and zero PCCF. Among the conventional sequences, maximal-length (m)-sequences have the smallest PACF side lobes -1. The disadvantage of these sequences is their poor PCCF which increases rapidly with their length ref. Consequently, m-sequences are optimal for single-point diagnosis systems. Large sets of sequences with relatively good PCCF such as Gold sequences can be generated from a pair of m-sequences called the preferred pair. Zero Correlation Zone (ZCZ) sequences have recently been introduced to the field of wire diagnosis. Their performance was evaluated in the case of simultaneous diagnosis of multiple wires in [9], distributed diagnosis of noisy wire networks in [8], and simultaneous diagnosis of shielded cable bundles  in [10]. The distinctive property of ZCZ sequences is that they have a zero-correlation zone in their PACF and PCCF, as shown in Fig. 2.

Fig. 2.  PACF (top) and PCCF (bottom) of ZCZ sequences of length N=512. Add units on axes, probably then need to include freq or Tc  Should we show the whole periodic axis, or focus in on the first 2-3 peaks in the top plot, and similar distances in the bottom (the ZCZ zone looks really little … but it's actually pretty big, can e make it in terms of meters, so we can see how wide it is for this specific frequency, and why that should work for the wire network?

ZCZ codes are significant, because if the zero-correlation zone width is chosen to be large enough, interference from other codes transmitting simultaneously can be eliminated. To do this, the zero-correlation zone must satisfy the following bound:

| Zo≥2 dv Tc | (13) |
| --- | --- |

Using a ZCZ sequence satisfying the above bound, (12) becomes

| Rrsi'τ=aoδl+pap δl-p | (14) |
| --- | --- |

where δl is the Kronecker delta defined as

| δl=01 | if l≠0 | (15) |
| --- | --- | --- |
|  | if l=0 |  |

In this case, both self-interference and mutual interference are eliminated. The drawback of these sequences is the tradeoff between the zero-correlation zone width and the number of sequences available for a specific sequence length. For more details, refer to Table 1 in [9]. Consequently, they are more suitable for distributed diagnostic systems where the number and location of sensors are judiciously chosen [11].

# Experimental Validation

To validate the effectiveness of properly chosen ZCZ codes for distributed sensing, we measure reflection from a simple Y-shaped wire network with a single sensor and code. We then add up to 16 simultaneous codes as interferers.  We compare m, Gold, and ZCZ codes.

## 1) Test Setup

We use a Rohde & Schwarz MXO5 oscilloscope with two waveform generators, as shown in Fig. 3. Three RG-58 coaxial cables with characteristic impedance 50Ω are used to form the Y- network. Generator 1 (Gen1) is connected to the Connector cable of length 30.5 m, which connects to the junction of Branch 1 (13.7 m) and Branch 2 (31.2 m). A BNC T-adapter captures the reflected signal and injects it into channel 1 of the oscilloscope. Another T-adapter is used to connect the cables as shown in Fig. 3. The signals used for testing can be either STDR or SSTDR. The STDR is a square wave modulated by a sequence (as in (1)), and the SSTDR is a sine wave modulated by the STDR signal. The sequences are of length N=511 for m and Gold sequences, and N=512 for ZCZ sequences. The chip rate is 25 MHz, giving a chip duration of Tc=40 ns. The sequence is up-sampled by a factor of Ns=10, so the sample rate is 250 MHz. The highest sampling rate available for this generator is slightly above 300 MHz.

Open circuit

30.5m

13.7m

31.2m

Sensor

Open circuit

Fig. 3.  Experimental setup for simultaneous diagnosis using S/SSTDR distributed reflectometry.  Add ABC etc. for locations of reflections, lengths, and names of the cables.  For the shorter cable, make it open, short, or matched OR Gen2.  Show Gen1 instead of sensor. Show reflection incoming to the junction from 13.7 and 31.2m wires.  Copy this wire network into Fig 1 (replacing “cable” and “wire network”). You can leave out some of the details there (and like the load, gen1,gen2,etc.) Label Branch 1 and 2

2) Single-point diagnosis of wire network

In this section, we consider reflection from the wire network in Fig. 3 using only Gen 1, transmitting an m-sequence. We expect m-sequences to be optimal for single-point diagnosis systems. The 25 MHz  STDR m-sequence has a length N=511 and is up-sampled to 250 MHz. The distance from Gen1 is calculated as d=VOP/(2x time), where VOP=2108 m/s is 2/3 the speed of light, and we consider the wave traveling to the end of the cable and back (hence the factor of 2 in the denominator). The signal amplitude is a=62.5 mV peak.

The diagnosis of the wired network can be in either continuous mode, where the periodic STDR signal transmits continuously and is correlated continuously, or in non-continuous mode, where only one period of the m-sequence is used. A detailed analysis can be found in [8]. The correlated reflection signatures of the wire network in Fig 3 to the two diagnosis modes are plotted in Fig. 4.

Fig. 4. STDR response from a Y-shaped wire network using non-continuous (aperiodic) and continuous (periodic) monitoring with an m-sequence. Label ABCD peaks

The reflection peaks in Fig. 4 are from the impedance discontinuities in the wired network. The first significant peak (A) corresponds to the junction location (30.5 m) where 1/3 of the signal energy is reflected to the injection point ao=20.83 mV. Im confused by what ao is? Peak B is from the end of Branch 1, at  44.2m. Peak C is from the end of Branch 2 at 61.7m. The response using periodic correlation is smoother than the one using aperiodic correlation, which is due to the continuous nature of the diagnostic system. When the periodic correlation is performed, only the even PACF (12) is computed. For a non-continuous diagnostic system, another type of correlation function called the odd periodic correlation function (OPACF) must also be considered. The effect of the OPACF can be seen in the fluctuation of the response using aperiodic correlation.

## View from Generator 1

The periodic correlated reflection response of the Y-shaped wire network in Fig. 3 using a single generator (Gen1) is shown in Fig. 5. The generator is matched (50Ω, the end of Branch 2 is an open circuit, and the end of Branch 1 is measured with three different loads -- matched (50 Ω), open, and short. Gen1 injects STDR signals based on m-, Gold, or ZCZ sequences. The test signal of amplitude is 62.5 mV is injected continuously, forming a periodic signal. The combination of this incident and associated periodic reflection signals are collected by the scope. This is correlated with the incident signal offline, which is obtained by connecting Gen1 directly to the scope.  The correlated signals are shown in Fig. 5. All sequence types perform well in this case with no interference, but the m-sequence is the best.

Fig. 5. STDR response of the Y-shaped wire network in Fig. 3 using m, Gold, or ZCZ sequences applied at Gen1. The end of Branch1 is matched, open, or short. Gen2 is not active. Label ABCD peaks

The A peak seen in Fig. 5 at x=0 is caused by the reflection at the first T-junction. The second peak (B) is from the second T-junction used to form the Y-network at 30.5 m. Peak C from the end of Branch 1 appears at 44.2 m, the combination of the connector cable and Branch 1. The positive peak at C is from the open, the negative from the short, and the lack of peak from the matched load. Finally, reflection peak D from the open end of Branch 2 appears at 61.7 m. Additional smaller reflections (not shown) follow, from multiple reflections within the network. A similar correlated reflection response from SSTDR is shown in Fig. 6.

Fig. 6. SSTDR response of the Y-shaped wire network in Fig. 3 using m, Gold, or ZCZ sequences applied at Gen1. The end of Branch1 is matched, open, or short. Gen2 is not active. Label ABCD peaks

## View from Generator 2

In the previous section, from just the point of view of Gen1, we cannot determine which peak (at 30.5 m and 61.7m) corresponds to the end of which Branch. To remove this ambiguity, another sensor (at Gen 2) is added. The same network in Fig. 3 is used, except that now we are sending a test signal from Gen2 (which is matched), and Gen 1 is replaced by either an open, short, or matched load. The correlated reflection is shown in Fig. 7. As in Fig. 5, the peak at x=0  is caused by the BNC T-adapter at ABCD. The second peak is caused by the T-adapter at B (31.2m?). At ABCD (44.2 m), we see a positive peak corresponding to an open where Gen1 used to be. This peak is added to the peak caused by the end of Branch 2, which has an open circuit.

Fig. 7. STDR response of the Y-shaped wire network in Fig. 3 using m, Gold, or ZCZ sequences applied at Gen2. The end where Gen1 used to be is matched, open, or short. Label ABCD peaks

3) Distributed diagnosis of wire network

We now evaluate a simultaneous and distributed diagnosis of the network. Both generators are sending signals in the network at the same time but using different sequences. Generator 1 sends one STDR signal while generator 2 sends multiple signals (up to 15) to simulate multiple interferers (other signals from other generators testing other parts of the network).

## View from Generator 1

The correlated reflection signatures measured at Gen1 (originally seen in Fig. 5) are shown in Fig. 8. Additional sequences (interferers) are sent into the system by Gen2. Up to 15 additional sequences (each with amplitude xxx) are added up, slightly delayed from each other, and sent into the system by Gen2. Significant interference is seen for measurements at Gen1 for m and Gold sequences, but little interference is seen for ZCZ sequences. It is important to note that the amplitude of the signals was chosen so that when constructive interference (the sum) of all 16 codes occurs, which will give 16 times the original amplitude, the function generator and scope would not clip this total amplitude. Also, the reason codes were added up out of sync by portions of a chip, is that there is more interference from this than from codes that are delayed by integer chip lengths.

(a)

(b)

(c)

Fig. 8. STDR response of the Y-shaped wire network in Fig. 3 using m, Gold, or ZCZ sequences applied at Gen1. Gen2 makes the end of Branch1 matched. Gen2 injects up to 15 interference signals that are (a) m-sequences, (b) Gold sequences, or (c) ZCZ sequences.  Instead of matched lets say 0 interferers. (a) doesn’t say m-sequences, but b and c say Gold and ZCZ sequences in the legends. We should be consistent either way. Label ABCD peaks

The interference signals have a significant impact on the diagnostic capability of the system, as shown in Fig. 8. Even two interferers are enough to cause ambiguity when m and Gold sequences are used. On the other hand, ZCZ sequences with their ideal correlation properties are not impacted by other interferers in this time/distance zone.  Somewhere above we should say what zone you designed these codes for, and how many codes could be possible for this zone. Similar results are seen for SSTDR in Fig. 9. With 14 interferers, m and Gold sequences have significant interference, but the ZCZ codes have very little.

Fig. 9. SSTDR response of the Y-shaped wire network in Fig. 3 using m, Gold, or ZCZ sequences applied at Gen1. Gen2 makes the end of Branch1 matched and injects 14 interference signals that are (a) m-sequences, (b) Gold sequences, or (c) ZCZ sequences.  Label ABCD peaks and add no interference test.

## View from Generator 2

In this case, we switch our view of the network from generator 1 to generator 2. Gen2 is testing with an STDR test signal, and Gen1 is sending 4 other interference signals. The reflection response is shown in Fig. 10.

Fig. 10. STDR response of the Y-shaped wire network in Fig. 3 using m, Gold, or ZCZ sequences applied at Gen2. Gen1 is now matched and injects 4 interference signals that are (a) m-sequences, (b) Gold sequences, or (c) ZCZ sequences.  Label ABCD peaks and add no interference test.

# Simulation Results

The results in section IV demonstrate the relative interference of ML, Gold, and ZCZ codes for distributed sensing. The specific example is done for xxx GHz, etc… We would like to be able to extend these results to other applications, with different frequencies, sampling rates, lengths of sequencies and zero correlation zones, numbers of interferers, etc. For this purpose, we have developed a simulation tool. Xxxx Evan describe howxxx

Fig. 11. Diagnosis of Y-shaped wire network using STDR using ML, Gold, or ZCZ sequences. Gen. 1 is testing and Gen. 2 is connected to Branch1 for testing and injecting 14 interference signals.

Fig. 12. Diagnosis of Y-shaped wire network using SSTDR using ML, Gold, or ZCZ sequences. Gen. 1 is testing and Gen. 2 is connected to Branch1 for testing and injecting 14 interference signals.

# Conclusion

This paper concentrates on enhancing the distrusted diagnosis of wire networks using the STDR technique. This was possible by using a generalized version of the TDV method, which presents two advantages: interference reduction and better accuracy. Numerical simulations were carried out to verify the analytical results obtained.

# Disclosure

Dr. C. M. Furse is a co-founder of LiveWire Innovation, Inc. which is commercializing SSTDR technology, and therefore she is disclosing a financial conflict of interest.

# Acknowledgment

The authors gratefully acknowledge financial support for this research by the Fulbright U.S. Scholar Program, which is sponsored by the U.S. Department of State. Its contents are solely the responsibility of the author and do not necessarily represent the official views of the Fulbright Program or the Government of the United States. The authors would also like to thank XX for loaning the XX and engineers X, Y, and Z for their assistance in guiding its use.

# Références

| [1] | F. Auzanneau, "Wire Troubleshooting And Diagnosis: Review And Perspectives," Progress In Electromagnetics Research B, vol. 49, pp. 253-279, 2013. |
| --- | --- |
| [2] | C. Furse and R. Haupt, "Down to the wire [aircraft wiring]," IEEE Spectrum, vol. 38, no. 2, pp. 34 - 39, 2001. |
| [3] | S. Kingston, E. Benoit, A. S. Edun, F. Elyasichamazkoti, D. E. Sweeney, J. B. Harley, P. K. Kuhn and C. M. Furse, "A SSTDR Methodology, Implementations, and Challenges," Sensors, vol. 21, no. 16, p. 5268, 2021. |
| [4] | C. M. Furse, M. Kafal, R. Razzaghi and Y.-J. Shin, "Fault Diagnosis for Electrical Systems and Power Networks: A Review," IEEE Sensors Journal, vol. 21, no. 2, pp. 888 - 906, 2021. |
| [5] | M. Hartebrodt and K. Kabitzsch, "Fault detection in fieldbuses with time domain reflectometry," in 2004 IEEE Africon. 7th Africon Conference in Africa, Gaborone, Botswana, 2004. |
| [6] | A. Lelong, L. Sommervogel, N. Ravot and M. O. Carrion, "Distributed Reflectometry Method for Wire Fault Location Using Selective Average," IEEE Sensors Journal , vol. 10, no. 2, pp. 300 - 310, 2010. |
| [7] | F. Auzanneau, N. Ravot and L. Incarbone, "Chaos Time Domain Reflectometry for Online Defect Detection in Noisy Wired Networks," IEEE Sensors Journal , vol. 16, no. 22, pp. 8027 - 8034, 2016. |
| [8] | M. Addad and A. Djebbari, "Spread Spectrum Sensing Based on ZCZ Sequences for the Diagnosis of Noisy Wired Networks," IEEE Sensors Journal, vol. 21, no. 2, pp. 914 - 920, 2021. |
| [9] | M. Addad and A. Djebbari, "Simultaneous Multiple Cable Fault Locating Using Zero Correlation Zone Codes," IEEE Sensors Journal, 2020. |
| [10] | M. Addad and A. Djebbari, "Spread spectrum reflectometry for the simultaneous diagnosis of shielded cable bundles," Nondestructive Testing and Evaluation, vol. 39, no. 4, p. 939–953, 2023. |
| [11] | W. Ben Hassen, F. Auzanneau, F. Pérès and A. Tchangani, "A Distributed Diagnosis Strategy using Bayesian Network for Complex Wiring Networks," IFAC Proceedings Volumes, vol. 45, no. 31, pp. 42-47, 2012. |

Mouad Addad received his Ph.D. degree in Telecommunications from Djillali Liabes University of Sidi Bel Abbes, Algeria, in 2018.  He joined the Telecommunications Department at the same University in 2018 where he is currently an Associate Professor. From 2017 to 2018, he was a visiting Ph.D. student at the Institute of Electronics, Microelectronics, and Nanotechnology (IEMN) in Valenciennes, France. In September 2024, he joined the Electrical and Computer Engineering Department at the University of Utah as a Visiting Scholar. His research interests include sequence design for communication systems, multiple access spread spectrum systems, channel estimation, and electrical system diagnosis using reflectometry measurements.

Evan J. Benoit received a B.S. degree in Nuclear Engineering Technology from Excelsior College in 2015. He received his B.S. and M.S. degrees in Electrical and Computer Engineering from the University of Utah in 2020 and a Ph.D. in Electrical Engineering from the University of Utah in 2023. His focus of study was on electromagnetics and transmission lines.

From 2005 to 2015, he was a submarine nuclear field electrician’s mate in the US Navy. He began working as a Research Assistant at the University of Utah, Salt Lake City UT during the summer of 2018. His research explored the applicability and validation of spread spectrum time domain reflectometry for measuring complex impedances and reflection coefficients on transmission lines.

After completing his Ph.D. in 2023, he became a Lecturing instructor at the University of Utah Asia Campus in Seoul, South Korea, where he teaches physics and electrical and computer engineering courses in the first two years of the ECE undergraduate program.

Cynthia M. Furse (M’85–SM’99–F’08) is a Distinguished Professor of Electrical and Computer Engineering at the University of Utah. She applies electromagnetics to sensing and communication in complex lossy media such as the human body. She is a Fellow of the IEEE, the National Academy of Inventors, and the Applied Computational Electromagnetics Society and has received numerous teaching and research awards including the 2020 IEEE Chen To Tai Distinguished Educator Award.

This paragraph of the first footnote will contain the date on which you submitted your paper for review.

Mouad Addad is with the Telecommunications and Digital Signal Processing Laboratory, Djillali Liabes University of Sidi Bel Abbes, SBA, Algeria, and a visiting scholar with the Electrical and Computer Engineering Department, University of Utah, Salt Lake City, UT 84112 USA (e-mail: mouad.addad@univ-sba.dz and u6062840@utah.edu)

Ali Djebbari is with the Telecommunications and Digital Signal Processing Laboratory, Djillali Liabes University of Sidi Bel Abbes, SBA, Algeria, (e-mail: adjebari2002@yahoo.fr)

Cynthia M. Furse is with the Electrical and Computer Engineering Department, University of Utah, Salt Lake City, UT 84112 USA (e-mail: cfurse@ece.utah.edu).